Daily Central Line Maintenance Checklist – Template

Patient Name/ID#: ____________________________________ Unit: _____________ Room/Bed: __________

Date: __________

Person Completing Form: Name _________________________

Date of initial line placement: _____________________________

Date implanted port accessed: _____________________________

Date injection caps last changed: __________________________

Date administration set and add-on devices last changed: __________________

Set used for:  Continuous Infusion             Intermittent Infusion

Date dressing last changed: __________________	Dressing type:  Gauze       	Clear

| Critical Steps | Yes | No | N/A | Notes/Comments |
| --- | --- | --- | --- | --- |
| Necessity assessed If no longer necessary, remove, indicating details of removal in the records (including date, location, and signature and name of operator undertaking removal). |  |  |  |  |
| Injection sites are covered by caps or valved connectors |  |  |  |  |
| Caps changed today |  |  |  |  |
| Implanted ports newly accessed today |  |  |  |  |
| Accessed with (indicate type and size of needle) |  |  |  |  |
| Insertion site without evidence of infection |  |  |  |  |
| Dressing intact and labeled properly |  |  |  |  |
| Dressing changed today |  |  |  |  |
| Catheter stabilized/no tension on line |  |  |  |  |
| Administration set replaced and labeled this time? |  |  |  |  |

Procedural Reminders

Suspected Infection

- If central venous catheter infection is strongly suspected, replace catheter and all intravenous fluids, tubing, and caps.

Hand Hygiene

- Clean hands immediately before and after each episode of patient contact using the correct hand hygiene technique. (Use World Health Organization “My 5 Moments for Hand Hygiene”.)

Cap Changes

- Sanitize caps with 2%chlorhexidine gluconate in 70% isopropyl alcohol before and after each use (“Scrub the Hub”).

- Change caps when necessary using sterile gloves and mask, that is, after administering blood and if there is visual observation of blood in the caps.

- Change caps no more often than 72 hours (or according to the manufacturer’s recommendations and whenever the administration set is changed).

Tubing Changes

- Replace administration sets and add-on devices no more frequently than every 96 hours, and at least every 7 days, after initiation of use, unless contamination occurs.

- Replace set and add-on devices within 24 hours of start of infusion if fluids that enhance microbial growth are infused (for example, fat emulsions combined with amino acids and glucose in three-in-one admixture or blood products infused separately).

- Change needleless components as often as the administration set and no more often than 72 hours.

Dressing Changes

- Change gauze dressing every 2 days, clear dressings every 7 days, unless dressing becomes damp, loosened, or visibly soiled then change.

- Use sterile gauze or sterile, transparent, semipermeable dressings.

- Perform catheter site care using 2% chlorhexidine gluconate in 70% isopropyl alcohol to clean the insertion site during dressing changes.